Chapter 4, Solution 95.

A dc voltmeter with a sensitivity of 20 k/V is used to find the Thevenin equivalent of a linear network. Readings on two scales are as follows:

- 0-10 V scale: 4 V

- 0-50 V scale: 5 V

Obtain the Thevenin voltage and the Thevenin resistance of the network.

Solution

Let  1/sensitivity  =  1/(20 k ohms/volt)  =  50 A

For the 0 – 10 V scale,

Rm  =  Vfs/Ifs  =  10/50 A  =  200 k ohms

For the 0 – 50 V scale,

Rm  =  50(20 k ohms/V)  =  1 M ohm

### Rm

#### RTh

##### VTh

+

VTh  =  I(RTh + Rm)

- A 4V reading corresponds to

I  =  (4/10)Ifs  =  0.4x50 A  =  20 A

VTh  =  20 A RTh + 20 A 250 k ohms

=  4 + 20 A RTh			(1)

(b)	A 5V reading corresponds to

I  =  (5/50)Ifs  =  0.1 x 50 A  =  5 A

VTh  =  5 A x RTh + 5 A x 1 M ohm

VTh  =  5 + 5 A RTh			(2)

From (1) and (2)

0  =  -1 + 15 A RTh  which leads to  RTh  =  66.67 k ohms

From (1),

VTh  =  4 + 20x10-6x(1/(15x10-6))  =  5.333 V