Chapter 5, Solution 3.

Determine the output voltage when 20 µV is applied to the inverting terminal of an op amp and +30 µV to its noninverting terminal. Assume that the op amp has an open-loop gain of 200,000.

Solution

v0 = Avd = A(v2 - v1)

= 2 x 105 (30 + 20) x 10-6 = 10V