# Chapter 5, Solution 18.

For the circuit, shown in Fig. 5.57, solve for the Thevenin equivalent circuit looking into terminals A and B.

#### 7.5 V

#### 10 k

+

+

#### 10 k

### 10

B

A

b

a

c

2.5

Figure 5.57

For Prob. 5.18.

Solution

Write a node equation at a.  Since node b is tied to ground, vb = 0.  We cannot write a node equation at c, we need to use the constraint equation, va = vb.  Once, we know vc, we then proceed to solve for Vopen circuit and Ishort circuit.  This will lead to VThev(t) = Vopen circuit and Requivalent = Vopen circuit/Ishort circuit.

[(va – 7.5)/10k] + [(va – vc)/10k] + 0 = 0

Our constraint equation leads to,

va = vb = 0 or vc = –7.5 volts

This is also the open circuit voltage (note, the op-amp keeps the output voltage at –5 volts in spite of any connection between A and B.  Since this means that even a short from A to B would theoretically then produce an infinite current, Requivalent = 0.  In real life, the short circuit current will be limited to whatever the op-amp can put out into a short circuited output.

VThev = –7.5 volts;  Requivalent = 0-ohms.