# Chapter 5, Solution 25.

Calculate vo in the op amp circuit of Fig. 5.63.

+

_

_

12 k

–

+

+

3.7 V	                        20 k           vo

–

Figure 5.63

For Prob. 5.25.

Solution

This is a voltage follower.  If v1 is the output of the op amp,

v1  = 3.7 V

vo = [20k/(20k+12k)]v1 = [20/32]3.7 = 2.312 V.