Chapter 5, Solution 82.

A five-bit DAC covers a voltage range of 0 to 7.75 V. Calculate how much voltage each bit is worth.

Solution

The maximum voltage level corresponds to

11111  =  25 – 1  =  31

Hence, each bit is worth 		(7.75/31)  =  250 mV