# Chapter 5, Solution 85.

In the op amp circuit of Fig. 5.104, find the value of R so that the power absorbed by the 10-kΩ resistor is 10 mW.  Take vs = 2V.

##### vS

+

### 10 k

+

#### R

#### 40 k

Figure 5.104

For Prob. 5.85.

Solution

This is a noninverting amplifier.

vo = (1 + R/40k)vs = (1 + R/40k)2

The power being delivered to the 10-kΩ give us

P = 10 mW = (vo)2/10k  or  vo =  = 10V

Returning to our first equation we get

10 = (1 + R/40k)2  or  R/40k = 5 – 1 = 4

Thus,				R = 160 kΩ.