Chapter 8, Solution 40.

The switch in the circuit of Fig. 8.88 is moved from position a to b at t = 0.  Assume that the voltage across the capacitor is equal to zero at t = 0 and that the switch is a make before break switch.  Determine i(t) for all t > 0.

Figure 8.88

For Prob. 8.40.

Solution

# At  t  =  0-,  vC(0)  =  0  and  iL(0)  =  i(0)  =  (6/(6 + 2))4  =  3A

For  t  >  0, we have a series RLC circuit with a step input as shown below.

#### 14

#### i

#### 24V

#### 2 H

#### 0.02 F

##### 12V

+

+

#### 6

+

#### v

o = 1/ =  1/ =  5

- =  R/(2L)  =  (6 + 14)/(2x2)  =  5

Since   =  o, we have a critically damped response.

v(t)  =  Vs + [(A + Bt)e-5t],   Vs  =  24 – 12  =  12V

v(0)  =  0  =  12 + A  or  A  =  -12

i  =  Cdv/dt  =  C{[Be-5t] + [-5(A + Bt)e-5t]}

i(0)  =  3  =  C[-5A + B]  =  0.02[60 + B]  or B  =  90

Thus,	i(t)  =  0.02{[90e-5t] + [-5(-12 + 90t)e-5t]}

i(t)  =  {(3 – 9t)e-5t} A