Chapter 10, Solution 4.

Compute vo(t) in the circuit of Fig. 10.53.

Figure 10.53

For Prob. 10.4.

Solution

##### V1

##### –j1 Ω

##### Ix

##### j1

##### 0.5Ix

##### Vo

##### 160º V

Step 1. 	Convert the circuit into the frequency domain and solve for the node voltage, V1, using analysis.  The find the current IC = V1/[1+(1/(j4x0.25)] which then produces Vo = 1xIC. Finally, convert the capacitor voltage back into the time domain.

Note that we represented 16sin(4t–10º)  volts by 160º V.  That will make our calculations easier and all we have to do is to offset our answer by a –10º.

Our node equation is [(V1–16)/j] – (0.5Ix) + [(V1–0)/(1–j)] = 0.  We have two unknowns, therefore we need a constraint equation.  Ix = [(16–V1)/j] = j(V1–16).  Once we have V1, we can find Io = V1 /(1–j) and Vo = 1xIo.

Step 2.		Now all we need to do is to solve our equations.

[(V1–16)/j] – [0.5j(V1–16] + [(V1–0)/(1–j)] = [–j–j0.5+0.5+j0.5]V1 +j16+j8 = 0 or

[0.5–j]V1 = –j24 or V1 = j24/(–0.5+j) = (2490º)/(1.118116.57º)

= 21.47–26.57º V.

Finally, Ix = V1/(1–j) = (21.47–26.57º) (0.707145º) = 15.18118.43º A and Vo = 1xIo = 15.18118.43º V or

vo(t) =  15.181sin(4t–10º+18.43º) = 15.181sin(4t–8.43º) volts.