# Chapter 10, Solution 41.

##### +

##### vo

##### vs

+

### 2

### 250 mF

Find vo for the circuit in Fig. 10.86 assuming that vs = [6 cos(2t) + 4 sin(4t)] V.

Figure 10.86

For Prob. 10.41.

Solution

We apply superposition principle.  We let

vo   =  v1 + v2

where v1 and  v2  are due to the sources 6cos2t and 4sin4t respectively.  To find  v1, consider the circuit below.

-j2

+

_

+

60o	                                                                      2           V1

–

V1=22-j2 V(6) = 3+j3 = 4.243∠45°

Thus,

v1(t) = 4.243cos(2t+45°) volts.

To get v2(t), consider the circuit below,

–j

+

_

+

40o  	2            V2

–

V2=22-j4=3.2+j11.6=3.578∠26.57° or

v2(t) = 3.578sin(4t+26.57°) volts.

Hence,

vo = [4.243cos(2t+45˚) + 3.578sin(4t+26.57˚)] volts.