# Chapter 10, Solution 61.

Find the Thevenin equivalent at terminals a-b of the circuit in Fig. 10.104.

4

a

Ix

20o A		–j3                                   1.5Ix

b

Figure 10.104

For Prob. 10.61.

Solution

Find the Thevenin equivalent at terminals a-b of the circuit in Fig. 10.104.

##### +

##### Voc

##### V1 and V2

### Isc

4

a

Ix

20o A		-j3                                   1.5Ix

b

Figure 10.104

For Prob. 10.61.

Solution

Step 1.		First we solve for the open circuit voltage using the above circuit and writing two node equations.  Then we solve for the short circuit current which only need one node equation.  For being able to solve for Voc, we need to solve these three equations,

–2 + [(V1–0)/(–j3)] + [(V1–Voc)/4] = 0 and

[(Voc–V1)/4] – 1.5Ix = 0 where Ix = [(V1–0)/(–j3)].

To solve for Isc, all we need to do is to solve these three equations,

–2 + [(V2–0)/(–j3)] + [(V2–0)/4] = 0, Isc = [V2/4] + 1.5Ix, and

Ix = [V2/–j3].

Finally, VThev = Voc and Zeq = Voc/Isc.

Step 2.		Now all we need to do is to solve for the unknowns.  For Voc,

Ix = j0.33333V1 and (0.25+(1.5)(j0.33333))V1 = 0.25Voc or

(0.25+j0.5)V1 = (0.55902∠63.43°)V1 = 0.25Voc or

V1 = (0.44721∠–63.43°)Voc which leads to,

(0.25+j0.33333)V1 – 0.25Voc = 2

= (0.41666∠+53.13°)(0.44721∠–63.43°)Voc – 0.25Voc

= (0.186335∠–10.3°)Voc – 0.25Voc = (0.183333–0.25–j0.033333)Voc

= (–0.066667–j0.033333)Voc = (0.074536∠–153.435°)Voc = 2 or

Voc = VThev = 26.83∠153.44° V = (–24+j12) V.

Now for Isc,

Isc = [V2/4] + 1.5Ix = (0.25+(1.5)(j0.33333))V2 = (0.25+j0.5)V2.

[(V2–0)/(–j3)] + [(V2–0)/4] = 2 = (0.25+j0.3333)V2

= (0.41667∠53.13°)V2 = 2 or V2 = 4.8∠–53.13°

Isc = (0.25+j0.5)V2 = (0.55901∠63.435°)(4.8∠–53.13°)

= 2.6832∠10.305° A

Finally,

Zeq = Voc/Isc = 26.833∠153.435°/2.6832∠10.305°

= 10∠143.13° Ω or = (–8+j6) Ω.