Chapter 10, Solution 86.

The schematic is shown below.   We insert three pseudocomponent PRINTs at nodes 1, 2, and 3 to print V1, V2, and V3, into the output file.  Assume that w = 1, we set Total Pts = 1, Start Freq = 0.1592, and End Freq = 0.1592.   After saving and simulating the circuit, we obtain the output file which includes:

FREQ			VM($N_0002)			VP($N_0002)

1.592 E-01		6.000 E+01			3.000 E+01

FREQ			VM($N_0003)			VP($N_0003)

1.592 E-01		2.367 E+02			-8.483 E+01

FREQ			VM($N_0001)			VP($N_0001)

1.592 E-01		1.082 E+02			1.254 E+02

Therefore,

V1  =  6030o V   V2  =  236.7-84.83o V   V3  =  108.2125.4o V