Chapter 10, Solution 88.

The schematic is shown below.  We insert IPRINT and PRINT to print Io and Vo in the output file.  Since w = 4, f  =   w/2  =  0.6366, we set Total Pts = 1,  Start Freq = 0.6366, and End Freq =  0.6366 in the AC Sweep box.  After simulation, the output file includes:

FREQ			VM($N_0002)			VP($N_0002)

6.366 E-01		3.496 E+01			1.261 E+01

FREQ			IM(V_PRINT2)		IP (V_PRINT2)

6.366 E-01		8.912 E-01				-8.870 E+01

Therefore,		Vo  =  34.9612.6o V,	 Io  =  0.8912-88.7o A

vo  =  34.96 cos(4t + 12.6o)V,   	io  =  0.8912cos(4t – 88.7o )A