Chapter 12, Problem 79.

A balanced three-phase generator has an abc phase sequence with phase voltage Van = 255 0 V.  The generator feeds an induction motor which may be represented by a balanced Y-connected load with an impedance of 12 + j5  per phase.  Find the line currents and the load voltages.  Assume a line impedance of 2  per phase.