Chapter 13, Solution 14.

Obtain the Thevenin equivalent circuit for the circuit in Fig. 13.83 at terminals

a-b.

##### 800˚ A

##### 20090˚ V

Figure 13.83

For Prob. 13.14.

Solution

To obtain VTh, convert the current source to a voltage source as shown below.

+

–

# 160 V

+

VTh

–

2

j2

a

b

+

–

# j200 V

# I

-j3

j8

j6

5

Note that the two coils are connected series aiding.

L  =  L1 + L2 – 2M

jL  =  j6 + j8 – j4  =  j10

Thus,				–j200 + (5 + j10 – j3 + 2)I + 160  =  0

I  =  (– 160 + j200)/ (7 + j7)

But, 				–j200 + (5 + j6)I – j2I + VTh  =  0

VTh  =  j200 – (5 + j4)I  =  j200 – (5 + j4)(–160 + j200)/(7 + j7)

= j200 – (6.403138.66)(256.12128.66)/(9.899545) = j200–165.661122.32

= j200 + 88.57 – j140 = 88.57 + j60 = 106.9834.115

VTh  =  106.9834.12 V

To obtain ZTh, we set all the sources to zero and insert a 1-A current source at the terminals

a–b as shown below.

+

Vo

–

2

j2

a

b

-j3

j8

j6

5

# 1 A

# I1

# I2

Clearly, we now have only a super mesh to analyze.

(5 + j6)I1 – j2I2 + (2 + j8 – j3)I2 – j2I1  =  0

(5 + j4)I1 + (2 + j3)I2  =  0			(1)

But,				I2 – I1  =  1  or  I2  =  I1 – 1			(2)

Substituting (2) into (1),		(5 + j4)I1 +(2 + j3)(1 + I1)  =  0

I1  =  –(2 + j3)/(7 + j7)

Now,		(5 + j6)I1 – j2I1 + Vo  =  0

Vo  =  –(5 + j4)I1  =  (5 + j4)(2 + j3)/(7 + j7)  =  (–2 + j23)/(7 + j7)  =  2.33250

Zeq  =  Vo/1  =  2.33250 Ω.