Chapter 13, Solution 15.

Find the Norton equivalent for the circuit in Fig. 13.84 at terminals a-b.

##### 10030˚ V

Figure 13.84

For Prob. 13.15.

Solution

### j10 Ω

+

–

### j5I2

##### j5(I1–I2)

##### j20 Ω

+

# I1

# I2

(a)

b

+

–

# 10030o

Isc

a

20

The first step is to replace the mutually coupled circuits with the equivalent circuits using dependent sources.  To obtain IN, short-circuit a–b as shown in Figure (a) and solve for Isc.

Now all we need to do is to write our two mesh equations.

Loop 1.		–10060˚ + 20I1 + j10(I1–I2) + j5I2 = 0 or (20+j10)I1 – j5I2 = 10060˚

or (4+j2)I1 – jI2 = 2030˚

Loop 2.		–j5I2 + j10(I2–I1) + j20I2 + j5(I1–I2) = 0 or –j5I1 + j20I2 = 0 or I1 = 4I2

Substituting back into the first equation, we get, (4+j2)4I2 – jI2 = 2030˚ or (16+j7)I2 = 2030˚.

Now to solve for I2 = Isc = IN = (2030˚)/(16+j7) = (2030˚)/(17.46423.63˚) = 1.14526.37˚ A.

### j10 Ω

+

–

### j5I2

##### j5(I1–I2)

##### j20 Ω

+

# I1

# I2

(b)

b

+

–

# 10030o

+

Voc

–

a

20

To solve for ZN = Zeq = Voc/Isc, all we need to do is to solve for Voc.  In circuit (b) we note that I2 = 0 and we get the mesh equation, –10030˚ + (20+j10)I1 = 0 or I1 = (10030˚)/(22.3626.57˚) = 4.4723.43˚ A.

Voc = j10I1 – j5I1 (induced voltage due to the mutual coupling) = j5I1 = 22.3693.43˚ V.

Zeq = ZN = (22.3693.43˚)/(1.14526.37˚) = 19.52587.06˚ Ω.

or [1.0014+j19.498] Ω.