Chapter 13, Solution 18.

Find the Thevenin equivalent to the left of the load Z in the circuit in Fig. 13.87.

Figure 13.87

For Prob. 1318.

Solution

Replacing the mutually coupled circuit with the dependent source equivalent we get,

120V

+

### –j4 Ω

#### j2

##### j5I2

##### j5Ω

–

+

### j20Ω

+

–

### j5I1

### j6 Ω

### 4

#### I1

#### I2

Now all we need to do is to find Voc and Isc.  To calculate the open circuit voltage, we note that I2 is equal to zero.  Thus,

–120 + (4 + j(–4+5+6))I1 = 0 or I1 = 120/(4+j7) = 120/(8.06226∠60.255°)

= 14.8842∠–60.255°.

Voc = VThev = j5I1 + (4+j6)I1 = (4+j11)I1

= (11.7047∠70.017°)(14.8842∠–60.255°) = 174.22∠9.76° V

To find the short circuit current (Isc = I2), we need to solve the following mesh equations,

Mesh 1

–120 + (–j4+j5)I1 – j5I2 + (4+j6)(I1–I2) = 0 or

(4+j7)I1 – (4+j11)I2 = 120						(1)

Mesh 2

(4+j6)(I2–I1) – j5I1 + j22I2 = 0 or –(4+j11)I1 + (4+j28)I2 = 0 or

I1 = (28.2843∠81.87°)I2/(11.7047∠70.0169°) = (2.4165∠11.853°)I2

Substituting this into equation (1) we get,

(8.06226∠60.255°)(2.4165∠11.853°)I2 – (4+j11)I2 = 120 or

[(19.4825∠72.108°) – 4 –j11]I2 = 120 and

[5.9855+j18.5403–4–j11]I2 = (1.9855+j7.5403)I2 = 120 or

I2 = Isc = 120/(7.79733∠75.248°) = 15.3899∠─75.248° A

Checking using MATLAB we get,

>> Z = [(4+7j) (-4-11j);(-4-11j) (4+28j)]

Z =

4.0000 + 7.0000i  -4.0000 -11.0000i

-4.0000 -11.0000i   4.0000 +28.0000i

>> V = [120;0]

V =

120

0

>> I = inv(Z)*V

I =

16.6551 -33.2525i		(I1)

3.9188 -14.8829i		(I2 = Isc) = 15.3902∠─75.248° (answer checks)

Finally,

Zeq = VThev/Isc = (174.22∠9.76°)/(15.3899∠─75.248°)

= (11.32∠85.01°) Ω