Chapter 13, Solution 23.

If M = 0.2 H and vs = 12 cos 10t V in the circuit of Fig. 13.92, find i1(t) and i2(t).  Calculate the energy stored in the coupled coils at t = 15 ms.

Figure 13.92

For Prob. 13.23.

Solution

=  10

0.5 H  converts to  jL1  =  j5 ohms

1 H  converts to  jL2  =  j10 ohms

0.2 H  converts to  jM  =  j2 ohms

25 mF  converts to  1/(jC)  =  1/(10x25x10-3)  =  –j4 ohms

The frequency-domain equivalent circuit is shown below.

##### j5

+

##### j2I2

##### j10

+

##### j2I1

### 5

#### –j4

##### 120

+

#### I1

#### I2

For mesh 1,		–12 + j5I1 + j2I2 + (–j4)(I1–I2) = 0 or jI1 + j6I2 = 12 or

I1 + 6I2 = –j12							(1)

For mesh 2,		(–j4)(I2–I1) + j10I2 + j2I1 + 5I2 = 0 or

j6I1 + (5+j6)I2 = 0						(2)

From (1),		I1  =  –j12 – 6I2

Substituting this into (2) produces,

j6(–j12–6I2) + (5+j6)I2 = 0 = 72 + (5+j6–j36)I2 or

(5–j30)I2 = (30.414∠–80.54°)I2 = –72 or I2 = 2.367∠–99.46° A

I1 = –j12 – 6( –0.38909–j2.3351) = 2.33454 + j(–12+14.0106)

= 2.33454 + j2.0106) = 3.081∠40.74° A

Checking using matrices,

16j6(5+j6)I1I2=-j120 which leads to I1I2=(5+j6)-6-j615+j6-j36-j120

I1 = [(72–j60)/(5–j30)] = (93.723∠–39.806°)/(30.414∠–80.538°) = 3.082∠40.73° A and

I2 = [–72/(30.414∠–80.54)] = 2.367∠–99.46° A

Thus,

i1(t) =  3.081cos(10t + 40.74) A,   i2(t)  =  2.367cos(10t – 99.46) A.

At  t  =  15 ms,			10t  =  10x15x10-3 = 0.15 rad  =  8.59

i1  =  3.081cos(49.33)  =  2.00789 A

i2  =  2.367cos(–90.87)  =  –0.03594 A

w  =  0.5(5)(2.00789)2 + 0.5(1)(–0.03594)2 – (0.2)(2.00789)(–0.03594)

= 10.079056 + 0.0006458 + 0.0144327 = 10.094 J.

3.081cos(10t + 40.74) A, 2.367cos(10t – 99.46) A, 10.094 J.