# Chapter 13, Solution 27.

Find the average power delivered to the 50- resistor in the circuit of Fig. 13.96.

8

+

_

40 cos(20t) V

50

1 H

2 H

10

0.5 H

Figure 13.96

For Prob. 13.27.

Solution

We apply mesh analysis to the circuit as shown below.

8

+

_

50

10

j10

40  0

I1

I2

j20

j40

I3

To make the problem easier to solve, let us have I3 flow around the outside loop as shown.

For mesh 1,

–40 + 8I1 + j20I1 – j10I2 = 0 or (8+j20)I1 – j10I2 = 40		(1)

For mesh 2,

j40I2 – j10I1 + 50(I2 + I3) = 0 or –j10I1 + (50+j40)I2 + 50I3 = 0	(2)

For mesh 3,

–40 + 10I3 + 50(I3 + I2) = 0 or 50I2 + 60I3 = 40			(3)

In matrix form, (1) to (3) become

>> Z=[(8+20i),-10i,0;-10i,(50+40i),50;0,50,60]

Z =

8.0000 +20.0000i        0 -10.0000i        0

0 -10.0000i  50.0000 +40.0000i  50.0000

0            50.0000            60.0000

>> V=[40;0;40]

V =

40

0

40

>> I=inv(Z)*V

I =

0.6354 - 1.5118i

0.0613 + 0.4682i

0.6156 - 0.3901i

Solving this leads to   I50 = I2 + I3 = 0.0613+0.6156 + j(0.4682–0.3901) = 0.6769+j0.0781 = 0.6814∠6.58° A or I50rms  |I50rms| = 0.6814/1.4142 = 481.8 mA.

The power delivered to  the 50- resistor is

P = (I50rms)2R = (0.4818)250 = 11.608 W.