Chapter 13, Solution 53.

Refer to the network in Fig. 13.118.

(a) Find n for maximum power supplied to the 200- load.

(b) Determine the power in the 200- load if n = 10.

Figure 13.118

For Prob. 13.53.

Solution

(a)	The Thevenin equivalent to the left of the transformer is shown below.

#### 8

+

##### 20 V

The reflected load impedance is   ZL’  =  ZL/n2  =  200/n2.

For maximum power transfer,       8  =  200/n2  produces  n  =  5.

- If  n  =  10,  ZL’  =  200/10  =  2  and  I  =  20/(8 + 2)  =  2

p  =  I2ZL’  =  (2)2(2)  =  8 watts.