Chapter 13, Solution 68.

In the ideal autotransformer of Fig. 13.130, calculate I1, I2, and Io.  Find the average power delivered to the load.

##### 12045 V

Figure 13.130

For Prob. 13.68.

Solution

This is a step-up transformer.

##### 10 + j40

##### 12045

+

##### +

##### v2

##### +

##### v1

##### 2 – j6

##### N2

##### N1

### I2

### I1

For the primary circuit,	12045  =  (2 – j6)I1 + v1  			(1)

For the secondary circuit,	v2  =  (10 + j40)I2  				(2)

At the autotransformer terminals,

v1/v2  =  N1/(N1 + N2)  =  200/280  =  5/7,

thus v2  =  7v1/5				(3)

Also,				I1/I2  =  7/5  or  I2  =  5I1/7			(4)

Substituting (3) and (4) into (2),	v1  =  (10 + j40)25I1/49

Substituting that into (1) gives	12045  =  (7.102 + j14.408)I1

I1  =  12045/16.06363.76  =  7.4706–18.76 A

I2  =  5I1/7  =  5.3361–18.76 A

Io  =  –I1 + I2  =  [–(5/7) + 1]I1  =  2I1/7  =  2.1345–18.76 A

p  =  |I2|2R  =  (5.3361)2(10)  =  284.7 W.