Chapter 13, Solution 83.

The schematic is shown below.  In the AC Sweep box, we set Total Pts  =  1, Start Freq  =  0.1592, and End Freq  =  0.1592.  After simulation, the output file includes

FREQ		IM(V_PRINT1)	IP(V_PRINT1)

1.592 E–01	1.080 E+00		3.391 E+01

FREQ		VM($N_0001)		VP($N_0001)

1.592 E–01	1.514 E+01		–3.421 E+01

i.e.	iX  =  1.0833.91 A, Vx  =  15.14–34.21 V.

This is most likely incorrect and needs to have the values of turns changed.  Clearly the turns ratio makes L2 = 400000 and L1 = 4000000.

Checking with hand calculations.

Loop 1.	–6 + 1I1 + V1 = 0 or I1 + V1 = 6					(1)

Loop 2.	–V2 – j10I2 + 8(I2–I3) = 0 or (8–j10)I2 – 8I3 – V2 = 0			(2)

Loop 3.	8(I3–I2) + 6I3 +2Vx +V3 = 0 or –8I2 + 14I3 + V3 + 2Vx = 0 but

Vx = 8(I2–I3), therefore we get 8I2 – 2I3 + V3 = 0			(3)

Loop 4.	–V4 + (4+j2)I4 = 0 or (4+j2)I4 – V4 = 0				(4)

We also need the constraint equations, V2 = 2V1, I1 = 2I2, V3 = 2V4, and I4 = 2I3.  Finally,

Ix = I2 and Vx = 8(I2 – I3).

We can eliminate the voltages from the equations (we only need I2 and I3 to obtain the required answers) by,

(1)+0.5(2) = I1 + (4–j5)I2 – 4I3 = 6 and

0.5(3) + (4) = 4I2 – I3 + (4+j2)I4 = 0.

2Vx

Next we use I1 = 2I2 and I4 = 2I3 to end up with the following equations,

(6–j5)I2 – 4I3 = 6 and 4I2 + (7+j4)I3 = 0 or I2 = –[(7+j4)I3]/4 = (–1.75–j)I3

= (2.01556∠–150.255°)I3

This leads to	(6–j5)(–1.75–j)I3 – 4I3 = (–10.5–5–4+j(8.75–6))I3 = (–19.5+j2.75)I3 = 6 or

I3 = 6/(19.69296∠171.973°) = 0.304677∠–171.973° amps

= –0.301692–j0.042545.

I2 = (–1.75–j)(0.304677∠–171.973°)

= (2.01556∠–150.255°)(0.304677∠–171.973°)

= 614.096∠37.772° mA = 0.48541+j0.37615

and I2 – I3 = 0.7871+j0.4187 = 0.89154∠28.01°.

Therefore,

Vx = 8(0.854876∠22.97°) = 7.132∠28.01° V

Ix = I2 = 614.1∠37.77° mA.

Checking with MATLAB we get A and X from equations (1) – (4) and the four constraint equations.

>> A = [1 0 0 0 1 0 0 0;0 (8-10j) -8 0 0 -1 0 0;0 8 -2 0 0 0 1 0;0 0 0 (4+2j) 0 0 0 -1;0 0 0 0 -2 1 0 0;1 -2 0 0 0 0 0 0;0 0 0 0 0 0 1 -2;0 0 -2 1 0 0 0 0]

A =

1.0000                  0                  0                  0             1.0000                  0                  0                  0

0             8.0000 -10.0000i  -8.0000                  0                  0            -1.0000                  0                  0

0             8.0000            -2.0000                  0                  0                  0             1.0000                  0

0                  0                  0             4.0000 + 2.0000i        0                  0                  0            -1.0000

0                  0                  0                  0            -2.0000             1.0000                  0                  0

1.0000            -2.0000                  0                  0                  0                  0                  0                  0

0                  0                  0                  0                  0                  0             1.0000            -2.0000

0                  0            -2.0000             1.0000                  0                  0                  0                  0

>> X = [6;0;0;0;0;0;0;0]

X =

6

0

0

0

0

0

0

0

>> Y = inv(A)*X

Y =

0.9708 + 0.7523i = I1 = 1.2817∠37.773° amps

0.4854 + 0.3761i = I2 = 614.056∠37.769° mA = Ix

-0.3017 - 0.0425i = I3 = 0.30468∠–171.982° amps

-0.6034 - 0.0851i = I4

5.0292 - 0.7523i = V1

10.0583 - 1.5046i = V2

-4.4867 - 3.0943i = V3

-2.2434 - 1.5471i = V4

Ix = 614.1∠37.77° mA

Finally, Vx = 8(I2 – I3) = 8(0.7871+j0.4186) = 8(0.891489∠28.01°)

= 7.132∠28.01° volts