Chapter 16, Solution 3.

The natural response of an RLC circuit is described by the differential equation

for which the initial conditions are v(0) = 20 V and dv(0)/dt = 0. Solve for v(t).

Solution

Step 1.		Transform the equation into the s-domain and solve for v(t).

s2V(s) – (dv(0–)/dt) – sv(0–) + 2sV(s) – 2v(0–) + V(s) = 0 or

(s2+2s+1)V(s) – 20s – 40 = 0 or V(s) = 20(s+2)/(s2+2s+1)

Step 2.		Perform a partial fraction expansion and solve for V(s).  Inverse transform into the time-domain and solve for v(t).

s2 + 2s + 1  =  0,  thus  s1,2  =    =  -1,  repeated roots.

V(s) = 20(s+2)/(s2+2s+1) = 20(s+2)/(s+1)2 = A/(s+1) + B/(s+1)2

As+A+B = 20s+40 or A = 20 and A+B = 40 = 20 + B or B = 40 – 20 = 20

Thus,

v(t)  =  [(20 + 20t)e-t]u(t) V