Chapter 16, Solution 7.

The step response of an RLC circuit is given by

Given that i(0) = 6 and di(0)/dt = 12, solve for i(t).

Solution

Step 1.		We start by transforming the equation into the s-domain.  We then solve for I(s).

s2I(s) – (di(0–)/dt) – si(0–) + 2sI(s) – 2i(0–) + 5 I(s) = 10/s or

s2I(s) – (12) – 6s + 2sI(s) – 2x6 + 5I(s) = 10/s = (s2+2s+5)I(s) – 6(s+4) or

(s2+2s+5)I(s) = [6(s2+4s)+10]/s or I(s) = [6(s2+4s)+10]/[s(s2+2s+5)]

Step 2.		We need to find the roots of (s2+2s+5) and then perform a partial fraction expansion and then transform back into the time domain and realize i(t).

s2 + 2s + 5, has the roots s1,2  =    =  –1j2

I(s) = [6(s2+4s)+10]/[s(s+1+j2)(s+1–j2)] = [A/s] + [B/(s+1+j2)] + [C/(s+1–j2)]

A = 10/5 = 2; B = [6(1+j4–4–4–j8)+10]/[(–1–j2)(–j4)] = [6(–7–j4)+10]/[–8+j4] =

(–32–j24)/[4(–2+j)] = 4(–8–j6)/[4(–2+j)] = 2(–4–j3)/(–2+j) =

[2(–4–j3)(–2–j)]/[(–2+j)(–2–j)] = 2(8–3+j6+j4)/5 = 2(1+j2); C =

[6(1–j4–4–4+j8)+10]/[(–1+j2)(j4)] = [6(–7+j4)+10]/[–8–j4] =

(–32+j24)/[4(–2–j)] = 4(–8+j6)/[4(–2–j)] = 2(–4+j3)/(–2–j) =

[2(–4+j3)(–2+j)]/[(–2–j)(–2+j)] = 2(8–3–j6–j4)/5 = 2(1–j2).

I(s) = [2/s] + [(2+j4)/(s+1+j2)] + [(2–j4)/(s+1–j2)]

= [2 + 4.4721e63.435(e–t–j2t) + 4.4721e–63.435(e–t+j2t)]u(t)

=[2 + 4.472e–t(e–j2t+63.44 + ej2t–63.44)]u(t)

or = [2 + 8.944e–t cos(2t – 63.44)]u(t) A