#### Chapter 16, Solution 19.

##### A

##### B

##### +

##### v(t)

–

#### 30

### 10

##### 20V

+

##### t=0

#### 4 H

### 0.25 F

The switch in Fig. 16.42 moves from position A to position B at t=0 (please note that the switch must connect to point B before it breaks the connection at A, a make before break switch).  Find v(t) for t >0.

Figure 16.42

For Prob. 16.19.

Solution

Step 1.		First find all the initial conditions and then transform into the s-domain.

##### A

##### B

##### V1

#### 30

### 10

### 4/s

##### 20V

+

##### t=0

#### 0.5/sIS

#### 4s

##### V2

Since the capacitor is not connected to a circuit, we do not know its initial condition so we can assume it is zero (v(0) = 0).  We can find iL(0) by letting the inductor be a short and iL(0) = 20/40 = 0.5 amp.

[(V1–0)/(4/s)] + [(V1–V2)/(4s)] + (0.5/s) = 0 and

[(V2–V1)/(4s)] + (–0.5/s) + [(V2–0)/10] = 0 where V = V1.  Next, add these together, [sV1/4] + [V2/10] = 0 or V2 = –2.5sV1.  Now we can solve for V1 and V.

Step 2.		[(s/4)+(1/(4s))+(2.5s/(4s))]V1 = –0.5/s

= [(s2+2.5s+1)/(4s)]V1 or V1 = –0.5(4)/(s2+2.5s+1) = –2/[(s+0.5)(s+2)]

= [–1.3333/(s+0.5)]+[1.3333/(s+2)] or

v(t) = [–1.3333e–t/2+1.3333e–2t]u(t) volts.