Chapter 16, Problem 28.

For the circuit in Fig. 16.51, find v(t) for t > 0.

4u(–t) A

100u(t) V

Figure 16.51

For Prob. 16.28.

Solution

+

##### 100/s

### 25/s

#### s

### 4

### 2

+

##### 24/s

+

#### V

#### I

Step 1.		Determine the initial conditions (at t = 0, the 4 amp current source turns off and the 100 volt voltage source becomes active).  Since the capacitor becomes an open circuit, iL(0) = 0 and v(0) = –4x6 = –24 volts.  Now convert the circuit into the s-domain and solve for V and then convert it into the time domain to obtain v(t),

Now for the mesh equation, [4+s+(25/s)+2]I–(24/s)–(100/s) = 0.  V = (25/s)I–24/s.

Step 2.		[(s2+6s+25)/s]I = 124/s or I = 124/(s2+6s+25) = 124/[(s+3+j4)(s+3–j4)] thus,

V = {3100/[s(s+3+j4)(s+3–j4)]}–24/s = [A/s]+[B/(s+3+j4)]+[C/(s+3–j4)] where

A = (3100/25)–24 = 124–24 = 100; B = 3100/[(–3–j4)(–j8)]

= 3100/[(5–126.87˚)(8–90˚)] = 77.5–143.13˚; and

C = 3100/[(j8)(–3+j4)] = 3100/[(5126.87˚)(890˚)] = 77.5143.13˚.

v(t) = [100+155e–3tcos(4t+143.13˚)]u(t) volts.