Chapter 16, Solution 40.

Given the network in Fig. 16.63, find v(t) for t > 0.

Figure 16.63

For Prob. 16.40.

Solution

##### 12/s

+

#### s

#### 6

##### +

##### V

#### 25/s

##### 8/s

+

Step 1.		First we determine initial conditions and then simplify the circuit and then transform it into the s-domain.  Just before the switch closes, the capacitor is an open circuit (iL(0) = 0) with v(0) = 4 –12 = –8 volts.

We can write a node equation at V and then solve for V.  Then we perform a partial fraction expansion and then solve for v(t).

[(V–(–12/s))/(s+6)]+[(V–(–8/s))/(25/s)] = 0.

Step 2.		[(1/(s+6))+s/25]V = [(s2+6s+25)/(25(s+6))]V

= –[12/(s(s+6))]–[8/25] = –[(12+0.32s2+1.92s)/(s(s+6))]

= –0.32[(s2+6s+37.5)/(s(s+6))] or

V = –8[(s2+6s+37.5)/(s(s+3+j4)(s+3–j4))]

= [A/s]+[B/(s+3+j4)]+[C/(s+3–j4)] where A = –8[37.5/25] = –12;

B = –8[((–3–j4)2+6(–3–j4)+37.5)/((–3–j4)(–j8)]

=  –8[(–7+j24–18–j24+37.5)/(–32+j24)]

= –8[(12.5)/(40143.13˚)] = 2.536.87˚; and

C = –8[((–3+j4)2+6(–3+j4)+37.5)/((–3+j4)(j8)]

= –8[(–7–j24–18+j24+37.5)/(–32–j24)]

= –8[(12.5)/(40–143.13˚)] = 2.5–36.87˚

V = [–12/s] + [2.536.87˚/(s+3+j4)] + [2.5–36.87˚/(s+3–j4)] or

v(t) = [–12+2.5e–3t(e–j4t+36.87˚+ej4–36.87˚)]u(t) amps

= [–12+5e–3t(cos(4t–36.87˚)]u(t) volts.