Chapter 16, Solution 41.

Find the output voltage vo(t) in the circuit of Fig. 16.64.

Figure 16.64

For Prob. 16.41.

Solution

### 5

##### 3/s

### s

##### 1/s

### 100/s

##### Vo

Step 1.		First we need to determine the initial conditions.  We see that vo(0) = 0 since the inductor becomes a short.  We also note that the initial current through the inductor is the same as the current through the 10 Ω resistor or iL(0) = [3(5x10)/(5+10)]/10 = 1 amp.  Then we simplify the circuit and convert it into the s-domain and solve for Vo.  We then perform a partial fraction expansion and convert into the time domain.

–[3/s] + [(Vo–0)/5] + [(Vo–0)/(100/s)] + [(Vo–0)/s] + [1/s] = 0.

Step 2.		[0.2+(s/100)+(1/s)]Vo = 2/s = [(s2+20s+100)/(100s)]Vo or

Vo = 200/[(s+10)2] and

vo(t) = [200te–10t]u(t) volts.