Chapter 16, Solution 42.

Given the circuit in Fig. 16.65, find i(t) and v(t) for t > 0.

12 V

Figure 16.65

For Prob. 16.42.

Solution

### 4/s

### s

### 1

##### 4/s

+

### 4/s

##### V

Step 1.		First we need to find the initial conditions.  Since the inductor becomes a short and the capacitor becomes an open circuit, all the current flows through the 1 Ω and 2 Ω resistors or i(0) = –12/3 = –4 amps and v(0) = 4x1 = 4 volts.  Next we need to convert the circuit into the s-domain and solve for V and I.  Once we have done that, we can perform partial fraction expansions and convert back into the time domain.

[(V–0)/1] + [(V–4/s)/(4/s)] + [(V–0)/s] – [4/s] = 0 and I = [(V–0)/s] – [4/s].

Step 2.		[(1+(s/4)+(1/s)]V = [(s2+4s+4)/(4s)]V = 1+4/s = (s+4)/s or

V = (4s+16)/[(s+2)2] = [A/(s+2)]+[B/(s+2)2] where As+2A + B = 4s+16 and

A = 4 and B = 16–2A =8.  I = [4/(s(s+2))]+[8/(s(s+2)2)]–4/s

The partial fraction expansion is straight forward for the first and third terms, but the second term takes a little work.  8/(s(s+2)2 = [a/s]+[b/(s+2)]+[c/(s+2)2] or as2+a4s+a4+bs2+b2s+cs = 8 or a = 2, b = –2, and c = –4.

Thus, I = [2/s]+[–2/(s+2)]+[2/s]+[–2/(s+2)]+[–4/(s+2)2]–4/s = –[4/(s+2)] – [4/(s+2)2] and we finally get,

v(t) = [4e–2t + 8te–2t]u(t) volts and

i(t) = [–4e–2t – 4te–2t]u(t) amps.