Chapter 16, Solution 51.

In the circuit of Fig. 16.74, find i(t) for t > 0.

50 V

Figure 16.74

For Prob. 16.51.

Solution

##### V

##### 50/s

+

#### 0.25s

#### 6

### 25/s

##### I

### 4

Step 1.		First we note that the initial conditions for the capacitor and inductor have to be equal to zero.  Next we simplify the circuit and then convert the circuit into the s-domain and solve for V.  Then we can solve for I and then perform a partial fraction expansion and convert I back into the time domain.

[(V–50/s)/(0.25(s+24))]+[s(V–0)/25]+[(V–0)/4] = 0 and I = [(0–V)/4] = –V/4.

Step 2.		[(4/(s+24))+(s/25)+0.25]V = [(s2+24s+6.25s+100+150)/(25(s+24))]V

= [(s2+30.25s+250)/(25(s+24))]V

= [{(s+15.125+j4.608)(s+15.125–j4.608)}/(25(s+24))]V = [200/(s(s+24))] or

V = 5,000/[s(s+15.125+j4.608)(s+15.125–j4.608)] and

I = –1250/[s(s+15.125+j4.608)(s+15.125–j4.608)] = [A/s]+[B/(s+15.125+j4.608)]

+[C/(s+15.125–j4.608)] where A = –1250/250 = –5;

B = –1250/[(–15.125–j4.608)(–j9.216)] = 1250180˚/[(15.811–163.06˚)(9.216–90˚)]

= 8.57873.06˚; and C = 1250180˚/[(15.811163.06˚)(9.21690˚)] = 8.578–73.06˚.

Thus, i(t) = [–5+8.578e–15.125t(e–j4.608t+73.06˚+ej4.608–73.06˚)]u(t) amps

i(t) = [–5+17.156e–15.125tcos(4.608t–73.06˚)]u(t) amps.