Chapter 16, Solution 52.

If the switch in Fig. 16.75 has been closed for a long time before t = 0 but is opened at t = 0, determine ix and vR for t > 0.

Figure 16.75

For Prob. 16.52.

Solution

Step 1.		Fist we need to determine the initial conditions.  Just before the switch opens, vC(0) = 16 volts and iL(0) = 2 amps.  Next we convert the circuit into the s-domain.

### 12

16/s

+

36/s

Ix

### s

##### 2/s

### 8

+

VR

–

We can now write a mesh equation (this time going in the counter-clockwise direction).  [s(Ix+2/s)]+[8Ix]+[12Ix]+[(36/s)Ix]+(16/s) = 0 and VR = –8Ix.

Step 2.		[s+8+12+(36/s)]Ix = [(s2+20s+36)/s]Ix = –2–16/s = –[2(s+8)/s] or

Ix = –2(s+8)/[(s+2)(s+18)] = [A/(s+2)]+[B/(s+18)] where

A = –2(–2+8)/(–2+18) = –2x6/16 = –0.75 and B = –2(–18+8)/(–18+2) = –1.25 thus,

ix(t) = [–0.75e–2t–1.25e–18t]u(t) amps and

vR(t) = –8ix(t) = [6e–2t+10e–18t]u(t) volts.