Chapter 16, Solution 53.

In the circuit of Fig. 16.76, the switch has been in position 1 for a long time but moved to position 2 at t = 0. Find:

- v(0+), dv(0+)/dt

- v(t) for t  0.

Figure 16.76

For Prob. 16.53.

Solution

Step 1.		Clearly iL(0) = 0 and v(0) = 4 volts.  When the switch moves to 2, iC(0+) = Cdv(0)/dt = –4/0.5 = –8 volts/second = 1dv(0)/dt.  Next we convert the circuit into the s-domain and solve for V.  Then we perform a partial fraction expansion and then convert back into the time domain.

0.5

4/s

+

1/s

+

V

0.25s

[(V–0)/(0.25s)]+[(V–0)/0.5]+[(V–4/s)s/1] = 0.

Step 2.		[(4/s)+2+s]V = [(s2+2s+4)/s]V = 4 or V = 4s/[(s+1+j1.7321)(s+1–j1.7321)]

= [A/(s+1+j1.7321)] + [B/(s+1–j1.7321)] where

A = 4(–1–j1.7321)/(3.464–90˚) = 4(2–120˚)/(3.464–90˚) = 2.309–30˚ and

B = 4(2120˚)/(3.46490˚) = 2.30930˚ or

v(t) = 2.309e–t[e–j1.7321t–30˚+ej1.7321t+30˚]u(t) volts or

v(t) = [4.618e–tcos(1.7321t+30˚)]u(t) volts.