Chapter 16, Solution 54.

The switch in Fig. 16.77 has been in position 1 for t < 0. At t =0, it is moved from position 1 to the top of the capacitor at t = 0. Please note that the switch is a make before break switch, it stays in contact with position 1 until it makes contact with the top of the capacitor and then breaks the contact at position 1.  Determine v(t).

Figure 16.77

For Prob. 16.54.

Solution

### 16

#### 4s

#### 2/s

##### +

##### V

#### I

### 16/s

Step 1.		First determine the initial conditions and then transform the circuit into the s-domain and solve for V.  Then perform a partial fraction expansion and then find v(t).  We will assume that the value of v(0) = 0.  iL(0) = 40/20 = 2 amps.

[16/s]I + [4s](I–2/s) + 16I = 0 and V = [16/s](–I).

Step 2.		[(16/s)+4s+16]I = [4(s2+4s+4)/s]I = 8 or

I = 8s/[4(s+2)2] = 2s/[(s+2)2] and V = –32/[(s+2)2]

v(t) = [–32te–2t]u(t) volts.