Chapter 16, Solution 60.

Find the response vR(t)  for t > 0 in the circuit in Fig. 16.83.  Let R = 3 , L = 2 H, and C = 1/18 F.

Figure 16.83

For Prob. 16.60.

Solution

##### 10/s

+

### 2s

#### 3

+

#### VR

### 18/s

##### V1

Step 1.		First convert the circuit into the s-domain.  Then use nodal analysis and eventually solve for VR, then perform a partial fraction expansion and convert back into the time domain.

[(V1-10/s)/3]+[(V1-0)/(18/s)]+[(V1-0)/(2s)] = 0 and VR = (10/s)-V1.

Step2.		[(1/3)+(s/18)+1/(2s)]V1 = 3.333/s = [(s2+6s+9)/(18s)]V1 or

V1 = 60/[(s+3)2] and VR = (10/s)-60/[(s+3)2].

Thus,

vR(t) = [10-60te-3t]u(t) volts.