Chapter 16, Solution 69.

Find I1(s) and I2(s) in the circuit of Fig. 16.92.

Figure 16.92

For Prob. 16.69.

Solution

#### 2 H

### 1 Ω

##### (10e–3t)u(t) V

+

#### i1

#### 2 H

### 1 Ω

#### i2

#### H

Find I1(s) and I2(s) in the following circuit.

Solution

### 1 Ω

#### I2

### 1 Ω

##### 10/(s+3)

+

#### I1

#### 2s

##### sI2

+

#### 2s

##### sI1

+

Step 1.		We note that the initial conditions in this case are equal to zero.  Next, we need to convert the circuit into the s-domain and use the model for mutually coupled circuits.  Then we can write the mesh equations and solve for I1 and I2.

Step 2.		–[10/(s+3)] + 2sI1 + sI2 + 1(I1–I2) = 0 and

1(I2–I1) + 2sI2 + sI1 + 1I2 = 0.  Simplifying we get,

(2s+1)I1 + (s–1)I2 = 10/(s+3) and (s–1)I1 + (2s+1)I2 = 0.

We can solve this directly using substitution or use matrices.  Let us use matrices.

The matrix inverse

Therefore,

I1 = 6.667(s+0.5)/[s(s+2)(s+3)] and

I2 = –3.333(s–1)/[s(s+2)(s+3)]

6.667(s+0.5)/[s(s+2)(s+3)], –3.333(s–1)/[s(s+2)(s+3)]