Chapter 17, Solution 67.

# The Schematic is shown below.  In the Transient dialog box, we type “Print step = 0.01s, Final time = 36s, Center frequency = 0.1667, Output vars = v(1),” and click Enable Fourier.  After simulation, the output file includes the following Fourier components,

FOURIER COMPONENTS OF TRANSIENT RESPONSE V(1)

DC COMPONENT =   2.000396E+00

HARMONIC   FREQUENCY    FOURIER    NORMALIZED    PHASE        NORMALIZED

NO         (HZ)     COMPONENT    COMPONENT    (DEG)       PHASE (DEG)

1     1.667E-01    2.432E+00    1.000E+00   -8.996E+01    0.000E+00

2     3.334E-01    6.576E-04    2.705E-04   -8.932E+01    6.467E-01

3     5.001E-01    5.403E-01    2.222E-01    9.011E+01    1.801E+02

4     6.668E-01    3.343E-04    1.375E-04    9.134E+01    1.813E+02

5     8.335E-01    9.716E-02    3.996E-02   -8.982E+01    1.433E-01

6     1.000E+00    7.481E-06    3.076E-06   -9.000E+01   -3.581E-02

7     1.167E+00    4.968E-02    2.043E-02   -8.975E+01    2.173E-01

8     1.334E+00    1.613E-04    6.634E-05   -8.722E+01    2.748E+00

9     1.500E+00    6.002E-02    2.468E-02    9.032E+01    1.803E+02

TOTAL HARMONIC DISTORTION =   2.280065E+01 PERCENT