# Saturday, October 27, 2018

# CHAPTER 1

P.P.1.1		A proton has 1.602 x 10-19 C.  Hence, 6.667 billion protons have

+1.602 x 10-19 x 6.667 x 109 = 1.6021 x 10–9 C

P.P.1.2		i = dq/dt = d(20–15t–10e–3t)/dt = (–15–10(–3)e–3t) mA

At   t = 1.0 sec,  i =  –15+30e–3 = –15+1.4936  =  –13.506 mA

P.P.1.3		q =    idt=018dt+128t2dt=8t01+83t312

= 8 + 8(8–1)/3 = 26.67 C

P.P.1.4		(a)	Vab  =  dw/dq  =  100/5  =  20 V

(b)	Vab  =  dw/dq  =  100/–10  =  –10 V

P.P.1.5		(a)	v = 6 i  =  30 cos (60  t)

p  =  v i  = 150 cos2 (60  t)

At  t = 5 ms,  p = 150 cos2 (60  5x10-3)  = 150 cos2 (0.3 )

=  51.82 watts

(b)	v  =  6 + 10 = 6 + 0t50cos (60 t) dt = 6 + 5060π  sin (60 t)

p =  vi =  5 cos (60 t)[6 + (50/(60 )) sin (60  t)]

At  t  = 5 ms,  p  =  5 cos (0.3){6 + (50/(60 )) sin (0.3 )}

=5(0.58779)(6+(0.26526)(0.80902)) = 18.264 watts

P.P.1.6		p  =  v i  =  115 x 12  =  1380 watts;  w  =  p x t

W = 1380x24 = 33.12 k watt-hours

P.P.1.7		p1 = 5(–45) = –225 w

p2 = 2(45) = 90 w

p3 = 0.12xI(20) = 0.6(25)(20) = 60 w

p4 = 3(25) = 75 w

Note that all the absorbed power adds up to zero as expected.

P.P.1.8		i = dq/dt =  e        =  –1.6 x 10-19 x 1013 = –1.6 x 10-6 A

p = v0 i = 25 x 103 x (1.6 x 10-6) = 40 mW

P.P.1.9		Minimum monthly charge 		= $12.00

First 100 kWh @ $0.16/kWh 		= $16.00

Next 160 kWh @ $0.10/kWh 	= $16.00

Remaining 0 kWh @ $0.06/kWh 	= $0.00

Total Charge	= $44.00

Average cost = $44/[100+160+0] = 16.923 cents/kWh

# P.P.1.10	This assigned practice problem is to apply the detailed problem solving technique to some of the more difficult problems of Chapter 1.