# Tuesday, November 13, 2018

# CHAPTER 2

P.P.2.1		i =  V/R  =  110/15  =  7.333 A

# P.P.2.2		(a)	v = iR =  3 mA[10 kohms] =  30 V

(b)	G = 1/R = 1/10 kohms = 100 µS

(c)	p = vi = 30 volts[3 mA] =  90 mW

# P.P.2.3 	p = vi which leads to  i = p/v = [30 cos2 (t) mW]/[15cos(t) mA]

or  i =   2cos(t) mA

R = v/i = 15cos(t)V/2cos(t)mA =  7.5 k

# P.P.2.4 	5 branches and 3 nodes.  The 1 ohm and 2 ohm resistors are in parallel.  The 4 ohm resistor and the 10 volt source are also in parallel.

P.P.2.5 	Applying KVL to the loop we get:

–32 + 4i – (–8) + 2i = 0 which leads to i =  24/6 = 4A

v1  =  4i  =  16 V    and    v2  =  –2i  =  –8 V

P.P.2.6 	Applying KVL to the loop we get:

–70 + 10i + 2vx + 5i = 0

But, vx = 10i and v0 = –5i.  Hence,

–70 + 10i + 20i + 5i = 0  which leads to i = 2 A.

Thus, vx  =  20V   and v0  =  –10 V

# P.P.2.7 	Applying KCL,  0 = –9 + i0  +  [i0 /4]  +  [v0 /8],  but  i0  =  v0/2

Which leads to:   9  =  (v0/2) + (v0/8) + (v0/8)  thus,  v0  =  12 V  and  i0  =  6 A

P.P.2.8

## 10V

+

–

–

+

2      i1              i3         4

## i2

8

## 6V

Loop 1

Loop 2

+  V1   –

+  V3   –

+

V2

–

At the top node,	0 = –i1 + i2 + i3 or i1 = i2 + i3		(1)

For loop 1		–10 + V1  + V2  =  0

or			V1  =  10  – V2 			(2)

For loop 2		– V2  + V3  – 6  = 0

or			V3  = V2  +  6    			(3)

Using (1) and Ohm’s law, we get

(V1/2) = (V2/8) + (V3/4)

and now using (2) and (3) in the above yields

[(10 – V2)/2] = (V2/8) + (V2 + 6)/4

or			[7/8]V2 = 14/4 or V2 = 4 V

V1 = 10 – V2 = 6 V, V3 = 4+6 = 10 V, i1 = (10–4)/2 = 3 A,

i2 = 4/8 = 500 mA, i3 = 2.5 A

# 3

# 4

# 5

# 4

# 3

# 6

# 4

# 3

P.P.2.9

Req

Combining the 4 ohm, 5 ohm, and 3ohm resistors in series gives 4+3+5 = 12.

But, 4 in parallel with 12  produces  [4x12]/[4+12] = 48/16 = 3ohm.

So that the equivalent circuit is shown below.

# Req

# 6

# 4

# 3

# 3

# 3

Thus, Req  = 4 + 3 + [6x6]/[6+6] = 10

# Req

# 16

# 18

# 9

# 20

# 20

# 1

# 2

# 5

# P.P.2.10

Combining the 9 ohm resistor and the 18 ohm resistor yields [9x18]/[9+18] = 6 ohms.

Combining the 5 ohm and the 20 ohm resistors in parallel produces [5x20/(5+20)] = 4 ohms  We now have the following circuit:

# 16

# 6

# 20

# 2

# 4

# 1

The 4 ohm and 1 ohm resistors can be combined into a 5 ohm resistor in parallel with a 20 ohm resistor.  This will result in [5x20/(5+20)] = 4 ohms and the circuit shown below:

# 2

# 4

# 16

# 6

The 4 ohm and 2 ohm resistors are in series and can be replaced by a 6 ohm resistor.  This gives a 6 ohm resistor in parallel with a 6 ohm resistor, [6x6/(6+6)] = 3 ohms.  We now have a 3 ohm resistor in series with a 16 ohm resistor or 3 + 16 = 19 ohms.  Therefore:

# Req =  19 ohms

# Geq

# Geq

# 6 S

# 12 S

# 8 S

# 2 S

# 4 S

# 4 S

8||4 = 8+4 = 12S

# 2||4 = 2+4 = 6S

P.P. 2.11

# Geq =  4 S

12 S in series with 6 S = {12x6/(12+6)] = 4 or:

# +

# v2

–

# 6

# +   v1     –

# i2

# 10

# 40

# 12

# 30V

–

+

# i1

# 30V

# +

# v2

–

# 8

-

+

# 4

# +   v1     –

P.P.2.12

6||12 = [6x12/(6+12)] = 4 ohm  and  10||40 = [10x40/(10+40)] = 8 ohm.

Using voltage division we get:

v1  = [4/(4+8)] (30) = 10 volts, v2 = [8/12] (30) =  20 volts

i1  = v1/12 = 10/12 = 833.3 mA,  i2 = v2/40 = 20/40 = 500 mA

P1  = v1 i1 = 10x10/12  = 8.333 watts, P2 = v2 i2  = 20x0.5 = 10 watts

# P.P.2.13

# i1

# i2

# 1k

# 20k

# 30mA

# 5k

# +

# v1

–

# +

# v2

–

# 4k

# 4k

# 30mA

# 3k

Using current division, i1  = i2 =  (30 mA)(4 kohm/(4 kohm + 4 kohm))  =  15mA

- v1  =  (3 kohm)(15 mA)  =  45 volts
v2  =  (4 kohm)(15 mA)  =  60 volts

- For the 3k ohm resistor, P1  = v1 x i1 =  45x15x10–3  =  675 mw
For the 20k ohm resistor, P2  =  (v2)2 /20k  =  180 mw

(c) 	 The total power supplied by the current source is equal to:
   	 P  = v2 x 10 mA  =  60x30x10–3  = 1.8 W

# P.P.2.14

Ra  =  [R1 R2 + R2 R3 + R3 R1]/ R1  =  [10x20 + 20x40 + 40x10]/10  =  140 ohms

Rb  =  [R1 R2 + R2 R3 + R3 R1]/ R2  =  1400/20  =  70 ohms

Rc  =  [R1 R2 + R2 R3 + R3 R1]/ R3  =  1400/40  =  35 ohms

P.P.2.15	We first find the equivalent resistance, R.  We convert the delta

sub-network to a wye connected form as shown below:

-

+

# i

# 240V

# b

# 13

# 24

# 30

# 20

# 10

# 50

# a

# a

# b

# 15

# 10

# 6

# 24

# 10

# 13

# c’

# b’

# a’

# n

Ra’n  =  20x30/[20 + 30 + 50]  =  6 ohms, Rb’n =  20x50/100  =  10 ohms

Rc’n =  30x50/100  =  15 ohms.

Thus, Rab =  13 + [(24 + 6)||(10 + 10)] + 15  =  28 + 30x20/(30 + 20)  =  40 ohms.

i = 240/ Rab =  240/40  =  6 amps

# P.P.2.16	For the parallel case, v  =  v0  =  110volts.
		p  =  vi  	i  =  p/v  =  40/110  =  364 mA

For the series case, v  = v0/N  =  110/10  =  11 volts
		i  =  p/v  =  40/11  =  3.64 amps

P.P.2.17	We use equation (2.61)

(a)	R1  =  50x10-3/ (1-10-3)  =  0.05/999 =  50 mΩ (shunt)

(b)	R2  =  50x10-3/(100x10-3 – 10-3)  =  50/99  =  505 mΩ (shunt)

(c)	R3  =  50x10-3/(10x10-3-10-3)  =  50/9  =  5.556 Ω (shunt)